1）The state diagram of the transmission module and receiving module

2) the corresponding Verilog code.

传输模块：

module uart_tx (

input clk,

input [7:0] tx_data,

input tx_start,

output tx_busy,

output reg txd

);

localparam IDLE = 0;

localparam START = 1;

localparam SENDING = 2;

localparam DONE = 3;

reg [2:0] state=IDLE, next_state=IDLE;

reg [2:0] count;

always @(posedge clk ) begin

state <= next_state;

end

always @(*) begin

next_state = state;

case (state)

IDLE: begin

if (tx_start) begin

next_state = START;

end

end

START: begin

next_state = SENDING;

end

SENDING: begin

if (count == 7) begin

next_state = DONE;

end else begin

next_state = SENDING;

end

end

DONE: begin

next_state = IDLE;

end

endcase

end

always @(posedge clk ) begin

if (state == SENDING) begin

count <= count + 1;

end else if (state == DONE||state == IDLE) begin

count <= 0;

end

end

always @(posedge clk ) begin

if (state == START) begin

txd <= 0;

end else if (state == SENDING) begin

txd <= tx_data[count];

end else if (state == DONE|state==IDLE) begin

txd <= 1;

end

end

assign tx_busy = (state == SENDING);

endmodule

接收模块：

module uart_rx (

input clk,

input rxd,

output  data_rdy,

output  [7:0] data

);

localparam IDLE = 0;

localparam RECEIVING = 1;

localparam DONE = 2;

reg [2:0] state=0, next_state=0;

reg [2:0] count;

reg [7:0] rx_data=0;

reg rx_data_rdy;

always @(posedge clk ) begin

state <= next_state;

end

always @(*) begin

next_state = state;

case (state)

IDLE: begin

if (rxd == 0) begin

next_state = RECEIVING;

end

end

RECEIVING: begin

if (count == 7) begin

next_state = DONE;

end else begin

next_state = RECEIVING;

end

end

DONE: begin

next_state = IDLE;

end

endcase

end

always @(posedge clk ) begin

if (state == IDLE) begin

count <= 0;

end else if (state == RECEIVING) begin

count <= count + 1;

end else if (state == DONE) begin

count <= 0;

end

end

always @(posedge clk ) begin

if (state == RECEIVING) begin

rx_data[count] <= rxd;

end

end

assign data_rdy = (state == DONE);  //

assign data = rx_data;  //

endmodule

顶层模块：

`include "uart_tx.v"

`include "uart_rx.v"

module top (

input clk,

input rxd,

output txd

);

wire [7:0] data;

wire dete_busy;

uart_tx  u_uart_tx (

.clk                     ( clk             ),

.tx_data                 ( data      [7:0] ),

.tx_start                ( dete_rdy        ),

.tx_busy                 ( dete_busy       ),

.txd                     ( txd             )

);

uart_rx  u_uart_rx (

.clk                     ( clk             ),

.rxd                     ( rxd             ),

.data_rdy                ( dete_rdy        ),

.data                    ( data      [7:0] )

);

endmodule

3）Simulate the transmission module to see check the timing of the output TxD signal is correct.

仿真文件代码：

//~ `New testbench

`timescale  1ns / 1ps

`include "uart_tx.v"

module tb_uart_tx;

// uart_tx Parameters

parameter PERIOD  = 10;

// uart_tx Inputs

reg   clk                                  = 0 ;

reg   [7:0]  tx_data                       = 0 ;

reg   tx_start                             = 0 ;

// uart_tx Outputs

wire  tx_busy                              ;

wire  txd                                  ;

initial

begin

forever #(PERIOD/2)  clk=~clk;

end

uart_tx  u_uart_tx (

.clk                     ( clk             ),

.tx_data                 ( tx_data   [7:0] ),

.tx_start                ( tx_start        ),

.tx_busy                 ( tx_busy         ),

.txd                     ( txd             )

);

initial

begin

$dumpfile("tb_uart_tx.vcd");

$dumpvars(0,tb_uart_tx);

tx_data = 8'b10011000;

tx_start = 1;

#20

tx_start = 0;

#200;

$finish;

end

endmodule

仿真波形如下：

结果正确，传输模块功能正常

4）Connect the transmission module to the receiving module to test whether the receiving module can work correctly. Copy and draw the simulation results to show the correctness of your design.

仿真代码如下：

//~ `New testbench

`include "top.v"

`timescale  1ns / 1ps

module tb_top;

// top Parameters

parameter PERIOD  = 10;

// top Inputs

reg   clk                                  = 0 ;

reg   rxd                                  = 0 ;

// top Outputs

wire  txd                                  ;

initial

begin

forever #(PERIOD/2)  clk=~clk;

end

top  u_top (

.clk                     ( clk   ),

.rxd                     ( rxd   ),

.txd                     ( txd   )

);

initial

begin

$dumpfile("tb_top.vcd");

$dumpvars(0,tb_top);

rxd = 1;

#100

rxd = 0;

#10

rxd = 1;

#10

rxd = 0;

#10

rxd = 1;

#10

rxd = 0;

#10

rxd = 0;

#10

rxd = 1;

#10

rxd = 1;

#10

rxd = 0;

#10

rxd = 1;

#200

$finish;

end

endmodule

仿真测试波形如下：

首先，仿真输入10表示信号开始接受，随后输入10100110 最后输入1表示结束。

从tx模块仿真信号看，rxd接受数据正常，数据成功从低到高位存入寄存器data中。

随后data_rdy信号拉高，传输模块开始按顺序传输数据，txd数据传输正常。

从tx和rx的state信号看，状态机状态跳转正常。

综上， uart_tx 和 uart_tx 模块功能正常。