1 实验目的和要求

To learn how to implement displaying a character string circularly by using FSM.

2 实验内容

- Synthesize a FSM circuit (i.e., draw the complete FSM circuit) to display the character string

2017 04 27 ddl

in one 7-segment LED circularly at a rate of 0.5 Hz for each digital. Blank the display (i.e., turn off all the segments of the LED) for the spaces in the string. Assume that your main clock frequency is 100MHz. Write the Verilog code for your design.

- Implement the above circuit in your EGO1/Basys3 board.

- Display the above character string in four 7-segment LED circularly at a rate of 0.5 Hz in your EGO1/Basys3 board

3 实验内容

（分点阐述实验步骤）

- 编写代码实现要求一。

- 上板验证。

- 修改代码实现要求三。

4 源代码与分析

本实验使用两种方式实现要求功能：

状态机方式代码：

功能一：

module ledplus(

input reset,

input clk,

output enable,

output reg [6:0]led

);

parameter CLKDIV=9999_9999;

reg [28:0] count=0;

reg [3:0] state,next_state;

//状态机状态参数

parameter s0=0,s1=1,s2=2,s3=3,s4=4,s5=5,s6=6,s7=7,s8=8,s9=9,s10=10,s11=11,s12=12,s13=13,s14=14;

//时钟分频

always @(posedge clk or negedge reset)begin

if(!reset) count<=0;

else begin

if(count==CLKDIV-1) count<=0;

else count<=count+1'b1;

end

end

//状态机

always @(posedge clk or negedge reset)begin

if(!reset) next_state<=s0;

else case (state)

s0:next_state<=s1;

s1:next_state<=s2;

s2:next_state<=s3;

s3:next_state<=s4;

s4:next_state<=s5;

s5:next_state<=s6;

s6:next_state<=s7;

s7:next_state<=s8;

s8:next_state<=s9;

s9:next_state<=s10;

s10:next_state<=s11;

s11:next_state<=s12;

s12:next_state<=s13;

s13:next_state<=s14;

s14:next_state<=s0;

endcase

end

//状态机更新

always @(posedge clk)begin

if(count==CLKDIV-1) state<=next_state;

end

//状态机输出

always @(posedge clk)begin

case(state)

s0:led<=7'b110_1101;

s1:led<=7'b111_1110;

s2:led<=7'b011_0000;

s3:led<=7'b111_0000;

s4:led<=7'b000_0000;

s5:led<=7'b111_1110;

s6:led<=7'b011_0011;

s7:led<=7'b000_0000;

s8:led<=7'b110_1101;

s9:led<=7'b111_0000;

s10:led<=7'b000_0000;

s11:led<=7'b011_1101;

s12:led<=7'b011_1101;

s13:led<=7'b000_1110;

s14:led<=7'b000_0000;

endcase

end

assign enable=1;

endmodule

功能二：

module ledplus(

input reset,

input clk,

output reg [3:0]enable,

output reg [6:0]led

);

reg [27:0] ledchoose;

reg [20:0] countdiv=0;

reg [1:0] countled=0;

parameter CLKDIV=9999_9999;

parameter COUNT_DIV=45_0000;

reg [28:0] count=0;

reg [3:0] state,next_state;

//状态机状态参数

parameter s0=0,s1=1,s2=2,s3=3, s4=4,s5=5,s6=6,s7=7, s8=8,s9=9,s10=10,s11=11,s12=12,s13=13,s14=14;

//时钟分频

always @(posedge clk or negedge reset)begin

if(!reset) count<=0;

else begin

if(count==CLKDIV-1) count<=0;

else count<=count+1'b1;

end

end

//状态机

always @(posedge clk or negedge reset)begin

if(!reset) next_state<=s0;

else case (state)

s0:next_state<=s1;

s1:next_state<=s2;

s2:next_state<=s3;

s3:next_state<=s4;

s4:next_state<=s5;

s5:next_state<=s6;

s6:next_state<=s7;

s7:next_state<=s8;

s8:next_state<=s9;

s9:next_state<=s10;

s10:next_state<=s11;

s11:next_state<=s12;

s12:next_state<=s13;

s13:next_state<=s14;

s14:next_state<=s0;

endcase

end

//状态机更新

always @(posedge clk)begin

if(count==CLKDIV-1) state<=next_state;

end

//状态机输出

always @(posedge clk)begin

case(state)

s0:ledchoose<=28'b0000000_0000000_0000000_0000000;

s1:ledchoose<=28'b0000000_0000000_0000000_1101101;

s2:ledchoose<=28'b0000000_0000000_1101101_1111110;

s3:ledchoose<=28'b0000000_1101101_1111110_0110000;

s4:ledchoose<=28'b1101101_1111110_0110000_1110000;

s5:ledchoose<=28'b1111110_0110000_1110000_1111110;

s6:ledchoose<=28'b0110000_1110000_1111110_0110011;

s7:ledchoose<=28'b1110000_1111110_0110011_1101101;

s8:ledchoose<=28'b1111110_0110011_1101101_1110000;

s9:ledchoose<=28'b0110011_1101101_1110000_0111101;

s10:ledchoose<=28'b1101101_1110000_0111101_0111101;

s11:ledchoose<=28'b1110000_0111101_0111101_0001110;

s12:ledchoose<=28'b0111101_0111101_0001110_0000000;

s13:ledchoose<=28'b0111101_0001110_0000000_0000000;

s14:ledchoose<=28'b0001110_0000000_0000000_0000000;

endcase

end

//时钟分频

always@(posedge clk or negedge reset)begin

if(!reset)countdiv<=0;

else begin

if(countdiv==COUNT_DIV-1 ) countdiv<=0;

else countdiv = countdiv+1'b1;

end

end

//led选择

always@(posedge clk or negedge reset)begin

if(!reset)countled<=0;

else begin

if(countdiv==COUNT_DIV-1)begin

if(countled==3) countled<=0;

else countled =countled+1'b1;

end

end

end

//led输出

always @(posedge clk)begin

case(countled)

2'b00:led<=ledchoose[27:21];

2'b01:led<=ledchoose[20:14];

2'b10:led<=ledchoose[13:7];

2'b11:led<=ledchoose[6:0];

endcase

end

//使能输出

always @(posedge clk)begin

case(countled)

2'b00:enable=4'b1000;

2'b01:enable=4'b0100;

2'b10:enable=4'b0010;

2'b11:enable=4'b0001;

endcase

end

endmodule

不使用状态机 通过移位方式实现功能：

`include "clk_division.v"

`include "segment.v"

module top

(

input clk_sys,

input mode,

input reset,

output [7:0] led,

output [3:0] segena

);

reg [63:0]bcd=64'hBBB2_017B_04B2_7DDE;//字符编码

reg [15:0] seg_selection;//段选

reg mode1;//模式选择

wire ena = 1'b1;

wire clk;   //0.5Hz时钟

wire clk1; // 20ms时钟

// 时钟分频

clk_division #(.F_CLK_SYS( 200_000_000 ) , .F_CLK_DIVISION( 1 ) ) clk_division_inst (

.ena( 1'b1 ),

.clk_in( clk_sys ),

.clk_out( clk )

);

// 时钟分频

clk_division #(.F_CLK_SYS( 100_000_000 ) , .F_CLK_DIVISION( 500 ) ) clk_division_inst1 (

.ena( 1'b1 ),

.clk_in( clk_sys ),

.clk_out( clk1 )

);

always @(posedge clk or negedge reset) begin

if (~reset) begin //复位

bcd<=64'hBBB2_017B_04B2_7DDE;

seg_selection <= 16'hBBBB;

end

else begin//单位显示

if (~mode) begin

if(mode1) begin

bcd <= 64'hBBB2_017B_04B2_7DDE;

mode1<=mode;

end

else begin

seg_selection <= bcd[63:48];

bcd <= {bcd[63:52],bcd[47:0],bcd[51:48]};

mode1<=mode;

end

end

else begin//四位显示

if(~mode1) begin

bcd <= 64'hBBB2_017B_04B2_7DDE;

mode1<=mode;

end

else begin

seg_selection <= bcd[63:48];

bcd <= {bcd[59:0],bcd[63:60]};

mode1<=mode;

end

end

end

end

// 数码管模块，输入时钟和seg_selection，输出数码管段选和位选信号

segment segment_inst (

.clk_sys(clk_sys),

.clk(clk1),

.reset(reset),

.Disp_Data(seg_selection),

.Sel(segena),

.led(led)

);

endmodule

5 实验结果和分析=

测试效果如下功能正常

6 实验结论

（基于实验结果和分析，得出结论）

成功使用两种方式编写Verilog代码，实现了lab所要求的功能。